Risk Assessment

Project: Skyline Roof Surveys (Mavic 3)

UAS Operator: Skyline Roof Surveys Ltd

Generated: 2026-07-09 · Template version 0.1.0

DRAFT — review required before operational use.

Baseline hazards for this operation, tailored to your aircraft type and launch/recovery method. Review, adjust residual-risk ratings, and add site-specific hazards before use.

| Hazard | Cause | Consequence | Mitigation | Residual | Review |
| --- | --- | --- | --- | --- | --- |
| Loss of control | Link loss, FC fault, pilot error | Impact with people/property | Pre-flight checks; lost-link procedure; emergency landing area; observer where used | Medium | review required |
| Flyaway | GNSS/compass error, link loss | Uncontrolled flight, airspace infringement | Compass/GNSS check; geofence where fitted; flyaway procedure 2.16.7 | Medium | review required |
| Battery fire | Damaged/over-discharged LiPo | Fire, injury | Battery inspection/storage policy; fire procedure 2.16.6 | Low | review required |
| Propeller injury | Contact with spinning props | Cuts/injury to crew | Arming discipline; prop guards where fitted; crew brief | Low | review required |
| Collision with people | Loss of control, incursion | Injury | A minimum horizontal separation of 50 m must be maintained from uninvolved persons, reducible to 30 m during take-off and landing. Assemblies of people must never be overflown, and a minimum horizontal separation of 50 m from assemblies must be kept (also observing the 1:1 rule: horizontal separation at least equal to the height of the aircraft). | Medium | review required |
| Collision with property | Obstacle, control error | Damage | Onsite survey; obstacle awareness; altitude limit | Low | review required |
| Poor weather | Wind/precip/visibility | Loss of control / lost VLOS | Weather limits 2.8.3; on-site re-check; abort criteria | Low | review required |
| GNSS failure | Interference, space weather | Degraded control | Non-GNSS mode; procedure 2.16.8 | Medium | review required |
| Command link loss | Range, interference | Lost-link failsafe | Range test; failsafe configured/tested; procedure 2.16.5 | Medium | review required |
| Public entering operating area | Unmonitored access point | Injury risk | Cordon 2.10; ground incursion 2.16.9; marshal | Low | review required |
| Data/privacy incident | Capture/handling of personal data | Privacy breach | Data & privacy procedure; retention limits | Low | review required |