Emergency Procedures

Project: Skyline Roof Surveys (Mavic 3)

UAS Operator: Skyline Roof Surveys Ltd

Generated: 2026-07-09 · Template version 0.1.0

DRAFT — review required before operational use.

Quick-reference emergency actions (mirrors Operations Manual §2.16–2.18 and the Annex 5 occurrence form). Action memory items first, then checklist. When in doubt, land.

## Lost link

Anticipate the configured failsafe; attempt link recovery (antenna, reduce range); prepare landing/termination area; monitor failsafe to completion.

## Flyaway

Note last position/heading/altitude and endurance; warn people nearby; contact ATC/airspace authority if controlled airspace/FRZ may be entered; report per ERP.

## Low battery

Return and land immediately with reserve; do not attempt to extend the flight.

## Loss of GPS/GNSS

Switch to non-GNSS mode; stabilise; increase separation; land. Do not rely on position-hold/RTH.

## Propulsion failure

Single motor: land immediately. Multiple: descend over clearest area.

## Control surface failure

Not applicable to this aircraft type.

## Payload detachment

Warn crew; land and make safe; do not fly over people; inspect before further flight.

## Crash

Make area safe; first aid; 999 if needed; isolate batteries; preserve evidence; report per ERP.

## Fire

Land/terminate away from people; do not approach burning LiPo; dry powder/sand; 999 if uncontained.

## Injury

Stop operations; administer first aid; call 999; notify accountable manager; complete occurrence form.

## Public incursion

Move aircraft clear and hold/land; never overfly; resume only when cordon restored.

## Weather deterioration

Cease manoeuvres; return and land at primary or emergency landing area.

## Aircraft entering adjacent area

Regain control; re-enter volume or land; report if separation/airspace limits infringed.

## Data/security incident

Contain; preserve records; assess personal-data impact; follow the data & privacy procedure.

## Occurrence & airprox reporting (Annex 5)

Occurrences must be reported in line with UK Reg (EU) 376/2014 and CAP 722; reportable accidents must be reported to the AAIB in line with UK Reg (EU) 996/2010.

- Occurrence details: date/time, location, aircraft, crew, description, contributing factors.

- Report reference: ____________________  (record the MORS / Airprox reference)